# Elevator System Design - 20-Floor Mixed-Use Building

## Problem Statement

Design elevator system for: 10 Residential floors (20 flats/floor, 500 people) + 4 Single-tenant office floors (400 people) + 6 Multi-tenant office floors (720 people) = 1,620 total occupants

## System Architecture

Core Components: Building → Scheduler → Elevator[] → Request Queue

Key Classes:

Elevator: Manages state (STATIONARY/MOVING/OUT_OF_ORDER), direction (UP/DOWN/IDLE), and two priority queues (upQueue, downQueue)

Scheduler: Implements LOOK algorithm, assigns requests to optimal elevator based on cost calculation

Request: Stores (sourceFloor, direction, destinationFloor, timestamp)

API:

requestElevator(floor, direction) → String requestId
selectDestination(elevatorId, floor) → void
getStatus(elevatorId) → ElevatorStatus

## Scheduling Algorithm: **LOOK** (Recommended)

Elevator moves in one direction serving all requests until last request, then reverses. Superior to FCFS (inefficient), SSTF (starvation risk), SCAN (unnecessary travel to end floors).

Assignment Logic:

Idle elevator → lowest cost (distance only)

Moving towards request (same direction) → low cost

Moving towards (opposite direction) → medium cost

Moving away → high cost

## Elevator Count Calculation

Given: Speed = 2.5 m/s, Capacity = 15 persons, Floor height = 3.5m, Door time = 6s

Round Trip Time (RTT): RTT = (2H/S) + (N_stops × T_door) + (Passenger transfer time)

Office zone (10 floors): RTT = 180s (with LOOK optimization)

Performance Metrics:

Interval = RTT / N_elevators (Target: 25-40s)

Handling Capacity = (300/RTT) × Capacity × N_elevators / Population (Target: >15%)

Calculations:

| Elevators | Office Interval | Office HC | Residential Interval | Residential HC | Status |
| --- | --- | --- | --- | --- | --- |
| 4 | 45s | 89% | 57s | 131% | ⚠️ Marginal |
| **5** | **36s** | **111%** | **51s** | **147%** | ✅ **Recommended** |
| 6 | 30s | 134% | 38s | 196% | ✅ Premium |

## Recommendation: **5 Elevators**

Technical Justification:

Meets performance targets: 36s office interval, 111% handling capacity

20% redundancy (system functional with 4 elevators if 1 fails)

Handles 500 residential + 1,120 office occupants effectively

LOOK algorithm prevents starvation, minimizes unnecessary movement

Alternative: 6 elevators for premium service (30s/38s intervals) if budget allows enhanced user experience for high-density residential.

Zoning Strategy (I haven’t seen this strategy in real life yet but I think it isn’t hard to implement and can be useful to optimize resources) :

Morning (8-10 AM): 3 elevators → office (1-10), 2 elevators → residential (11-20)

Evening (5-8 PM): 2 elevators → office departure, 3 elevators → residential arrival

Midday: All 5 elevators serve all floors dynamically

Scalability: Architecture supports 10-15% population growth, can add 6th elevator if needed. Supports predictive scheduling, dynamic zoning, and fault tolerance (graceful degradation on failure).