# Elevator System Design

## Technical Design for 20-Floor Mixed-Use Building

## Table of Contents

[Problem Statement](#problem-statement)

[Requirements Analysis](#requirements-analysis)

[Actors and Use Cases](#actors-and-use-cases)

[Capacity Planning](#capacity-planning)

[High-Level System Design](#high-level-system-design)

[Low-Level Object-Oriented Design](#low-level-object-oriented-design)

[Scheduling Algorithms](#scheduling-algorithms)

[Elevator Count Determination](#elevator-count-determination)

[Technical Recommendations](#technical-recommendations)

[Scalability and Optimizations](#scalability-and-optimizations)

## 1. Problem Statement

Design a comprehensive elevator system for a 20-floor mixed-use building with the following characteristics:

Building Composition:

10 Residential Floors (20 flats per floor)

4 Single-Tenant Office Floors (entire floor per company)

6 Multi-Tenant Office Floors (multiple offices per floor)

Design Goals:

Determine optimal number of elevators required

Design system architecture (high-level and low-level)

Implement efficient scheduling algorithm

Ensure acceptable service quality during peak hours

Design for scalability and fault tolerance

## 2. Requirements Analysis

### 2.1 Functional Requirements

### Individual Elevator Requirements:

State Management

- Three states: STATIONARY, MOVING, OUT_OF_ORDER

- Two movement directions: UP, DOWN

- Idle state when no requests

Control Systems

- External Control: UP/DOWN buttons on each floor

- Internal Control: Floor selection panel inside elevator

- Request queuing and prioritization

Door Operations

- Doors only open when elevator is stationary

- Automatic door control with configurable timeout

- Manual door open/close buttons (safety override)

- Obstruction detection

Display System

- Current floor indicator (internal and external)

- Direction indicator (up/down arrows)

- Queue status display

Safety Features

- Emergency stop button

- Emergency communication system

- Overload detection

- Fire service mode

### System-Level Requirements:

Request Management

- Queue multiple simultaneous requests

- Prioritize requests based on scheduling algorithm

- Handle both external (floor) and internal (destination) requests

Intelligent Scheduling

- Efficient elevator assignment algorithm

- Minimize average wait time

- Maximize throughput during peak hours

Monitoring & Control

- Real-time elevator position tracking

- System health monitoring

- Performance metrics collection

### 2.2 Non-Functional Requirements

### Performance Requirements:

| Metric | Target | Quality Rating |
| --- | --- | --- |
| **Average Wait Time** | < 30 seconds | Good service |
| **Interval** | 25-40 seconds | Acceptable to excellent |
| **Handling Capacity** | 15-20% in 5 min | Industry standard for offices |
| **System Uptime** | > 99.9% | High availability |

### Scalability Requirements:

Support 10-20% population growth without system redesign

Easy addition of new elevators to the system

Modular architecture allowing component upgrades

### Reliability Requirements:

Redundancy: System remains operational if one elevator fails

Failover: Automatic request redistribution on elevator failure

Graceful Degradation: Performance degrades smoothly under high load

### Safety & Compliance:

Accessibility standards compliance (Harmonized Guidelines 2016)

Fire safety (National Building Code of India)

Emergency power backup for at least 2 elevators

Indian Standard IS 14665 for Electric Lifts

### 2.3 Design Constraints

Building Height: 20 floors × 3.5m = 70 meters

Floor Height: 3.5 meters (standard)

Elevator Speed: 2.5 m/s (suitable for mid-rise)

Elevator Capacity: 1,600 kg (21 persons rated, ~15 practical capacity)

Door Operation Time: ~6 seconds (3s open + 3s close)

## 3. Actors and Use Cases

### 3.1 System Actors

### Primary Actors:

1. Passenger

Requests elevator from floor (external control)

Selects destination floor (internal control)

Uses emergency features when needed

Observes display for elevator status

2. System (Automated Controller)

Schedules elevator movements

Assigns requests to optimal elevators

Manages request queues

Controls displays and indicators

Handles emergency situations

### Secondary Actors:

3. Maintenance Personnel

Sets elevator to OUT_OF_ORDER state

Performs diagnostics

Monitors system health metrics

### 3.2 Use Cases

### UC-1: Request Elevator (External)

Primary Actor: Passenger

Trigger: Passenger presses UP or DOWN button on floor

Flow:

Passenger presses direction button at source floor

Request (sourceFloor, direction) added to system queue

Scheduler evaluates all elevator states

System assigns best available elevator using scheduling algorithm

Assigned elevator adds floor to its route

Elevator moves to source floor

Doors open, passenger enters

Success: Passenger inside elevator

Alternative Flows:

4a. All elevators at capacity → Request queued until elevator available

4b. Emergency mode active → Only firefighter elevator responds

### UC-2: Select Destination Floor (Internal)

Primary Actor: Passenger

Trigger: Passenger inside elevator presses floor button

Flow:

Passenger presses destination floor button

Request (destinationFloor) added to elevator's internal queue

Elevator adds floor to appropriate queue (UP or DOWN)

Elevator serves floors in optimized order

Elevator reaches destination floor

Doors open

Success: Passenger exits

### UC-3: Determine Elevator Movement

Primary Actor: System

Trigger: Multiple pending requests exist

Flow:

System receives multiple requests

For each request, evaluate all elevators:

- Current floor position

- Current direction

- Current load

- Pending requests in queue

Calculate "cost" for each elevator to serve request

Assign request to elevator with minimum cost

Success: Optimal elevator assigned

Algorithm Considerations:

Idle elevators: Lowest cost

Moving towards request (same direction): Low cost

Moving towards request (opposite direction): Medium cost

Moving away from request: High cost

### UC-4: Handle Emergency

Primary Actor: Passenger / System

Trigger: Emergency button pressed OR fire alarm

Flow:

Emergency signal detected

System triggers emergency protocol:

- Fire mode: Elevators return to ground floor, disable

- Emergency stop: Halt at nearest floor

Emergency communication activated

Alert building management system

Success: Safe state achieved

## 4. Capacity Planning

### 4.1 Population Analysis

### Residential Floors (10 floors):

Flats per floor: 20

Occupancy per flat: 2.5 persons

Total Residential: 10 × 20 × 2.5 = 500 persons

### Single-Tenant Office Floors (4 floors):

Floor area: 1,400 sq m per floor

Density: 14 sq m/person (standard office)

Total: 4 × 100 = 400 persons

### Multi-Tenant Office Floors (6 floors):

Floor area: 1,400 sq m per floor

Density: 11.7 sq m/person (higher density)

Total: 6 × 120 ≈ 720 persons

Total Building Population: 500 + 400 + 720 = 1,620 persons

### 4.2 Traffic Patterns

Different user groups have distinct traffic patterns:

| Time Period | Type | Population | Peak Demand (5-min) |
| --- | --- | --- | --- |
| **8:30-9:30 AM** | Morning Office Up-Peak | 1,120 office | 17-20% (190 pax) |
| **12:00-1:00 PM** | Lunch Hour (bi-directional) | 1,120 office | 10-12% (112 pax) |
| **5:00-6:00 PM** | Evening Office Down-Peak | 1,120 office | 17-20% (190 pax) |
| **6:00-8:00 PM** | Residential Up-Peak | 500 residential | 12-15% (60 pax) |

Critical Design Scenarios:

Morning Up-Peak (8:30-9:30 AM):

- Concentrated office arrivals

- Realistic peak 5-min demand: 112 passengers (25% of hourly flow)

- Primary concern: Office floors (1-10)

Evening Residential Up-Peak (6:00-8:00 PM):

- Residents returning home

- Peak 5-min demand: 60 passengers

- Distributed over 2-hour window

- Primary concern: Residential floors (11-20)

### 4.3 Floor Distribution Strategy

Floor 20 ┐
Floor 19 │
Floor 18 │ RESIDENTIAL ZONE (10 floors)
...      │ - 500 occupants
Floor 12 │ - Evening up-peak dominant
Floor 11 ┘

Floor 10 ┐
Floor 9  │ MULTI-TENANT OFFICE (6 floors)
Floor 8  │ - 720 occupants
Floor 7  │ - High traffic density
Floor 6  │ - Lunch hour peaks
Floor 5  ┘

Floor 4  ┐
Floor 3  │ SINGLE-TENANT OFFICE (4 floors)
Floor 2  │ - 400 occupants
Floor 1  ┘ - Concentrated arrivals/departures

Ground Floor (Lobby)

## 5. High-Level System Design

### 5.1 System Architecture

┌─────────────────────────────────────────────────────────────┐
│                    ELEVATOR CONTROL SYSTEM                   │
├─────────────────────────────────────────────────────────────┤
│                                                              │
│  ┌────────────────────────────────────────────────────┐    │
│  │           BUILDING MANAGEMENT LAYER                │    │
│  │  - Building Configuration                          │    │
│  │  - Elevator Fleet Management                       │    │
│  │  - Performance Monitoring                          │    │
│  └──────────────────┬─────────────────────────────────┘    │
│                     │                                        │
│  ┌──────────────────▼─────────────────────────────────┐    │
│  │         SCHEDULING & DISPATCH LAYER                │    │
│  │  ┌──────────────────────────────────────────────┐  │    │
│  │  │  Request Queue Manager                       │  │    │
│  │  │  - External requests (floor, direction)      │  │    │
│  │  │  - Internal requests (destination)           │  │    │
│  │  └──────────────────────────────────────────────┘  │    │
│  │  ┌──────────────────────────────────────────────┐  │    │
│  │  │  Scheduler (LOOK Algorithm)                  │  │    │
│  │  │  - Elevator assignment logic                 │  │    │
│  │  │  - Route optimization                        │  │    │
│  │  │  - Load balancing                            │  │    │
│  │  └──────────────────────────────────────────────┘  │    │
│  └──────────────────┬─────────────────────────────────┘    │
│                     │                                        │
│  ┌──────────────────▼─────────────────────────────────┐    │
│  │              ELEVATOR CONTROL LAYER                │    │
│  │                                                     │    │
│  │  ┌──────────┐  ┌──────────┐  ┌──────────┐         │    │
│  │  │ Elevator │  │ Elevator │  │ Elevator │  ...    │    │
│  │  │    1     │  │    2     │  │    3     │  (5-6)  │    │
│  │  ├──────────┤  ├──────────┤  ├──────────┤         │    │
│  │  │ State    │  │ State    │  │ State    │         │    │
│  │  │ Queue    │  │ Queue    │  │ Queue    │         │    │
│  │  │ Position │  │ Position │  │ Position │         │    │
│  │  └──────────┘  └──────────┘  └──────────┘         │    │
│  │                                                     │    │
│  └─────────────────────────────────────────────────────┘    │
│                                                              │
│  ┌──────────────────────────────────────────────────────┐  │
│  │              PHYSICAL INTERFACE LAYER                │  │
│  │  - Floor Buttons (External Control)                  │  │
│  │  - Cabin Buttons (Internal Control)                  │  │
│  │  - Displays (Floor indicators, Direction)            │  │
│  │  - Door Mechanisms                                    │  │
│  │  - Motor Control                                      │  │
│  └──────────────────────────────────────────────────────┘  │
└─────────────────────────────────────────────────────────────┘

### 5.2 Component Responsibilities

### 5.2.1 Building Manager

Maintains building configuration (floors, elevators)

Provides elevator fleet information to scheduler

Collects performance metrics

Handles elevator status changes (operational, out-of-order)

### 5.2.2 Request Queue Manager

Accepts external requests: (floor, direction)

Accepts internal requests: (elevatorId, destinationFloor)

Maintains separate UP and DOWN request queues

Removes fulfilled requests

### 5.2.3 Scheduler

Core responsibility: Assign requests to optimal elevators

Implements LOOK algorithm (recommended)

Evaluates elevator states before assignment

Balances load across elevator fleet

Handles edge cases (all elevators full, emergency mode)

### 5.2.4 Elevator Controller (per elevator)

Manages individual elevator state machine

Maintains UP and DOWN queues (sorted by floor)

Controls movement and door operations

Reports status to scheduler

### 5.3 Data Flow

┌─────────────────────────────────────────────────────────────┐
│                  REQUEST PROCESSING FLOW                     │
└─────────────────────────────────────────────────────────────┘

1. USER ACTION
   Passenger presses UP button on Floor 5
   │
   ▼
2. REQUEST CREATION
   Request{floor: 5, direction: UP, timestamp: T}
   │
   ▼
3. QUEUE INSERTION
   RequestQueue.addToUpQueue(request)
   │
   ▼
4. SCHEDULER EVALUATION
   For each elevator:
     - Check current state (STATIONARY, MOVING, OUT_OF_ORDER)
     - Calculate distance to Floor 5
     - Check current direction
     - Calculate cost/score
   │
   ▼
5. ELEVATOR ASSIGNMENT
   Best elevator selected (e.g., Elevator 3)
   Elevator3.addStop(5)
   │
   ▼
6. ELEVATOR MOVEMENT
   Elevator moves according to LOOK algorithm:
   - Serves all stops in current direction
   - Picks up Floor 5 passenger
   │
   ▼
7. INTERNAL REQUEST
   Passenger presses Floor 15
   Elevator3.addStop(15)
   │
   ▼
8. DESTINATION REACHED
   Elevator arrives at Floor 15
   Doors open, passenger exits
   Request fulfilled and removed from queue

### 5.4 API Design

### 5.4.1 External API (Public Interface)

/**
 * Main interface for elevator system interaction
 */
public interface IElevatorSystem {

    /**
     * Request elevator from a floor
     * @param floor Source floor number (0-20)
     * @param direction Desired direction (UP/DOWN)
     * @return Request ID for tracking
     */
    String requestElevator(int floor, Direction direction);

    /**
     * Select destination floor from inside elevator
     * @param elevatorId Which elevator the passenger is in
     * @param destinationFloor Where passenger wants to go
     */
    void selectDestination(int elevatorId, int destinationFloor);

    /**
     * Get current status of an elevator
     * @param elevatorId Elevator identifier
     * @return Current status object
     */
    ElevatorStatus getStatus(int elevatorId);

    /**
     * Emergency stop
     * @param elevatorId Elevator to stop
     */
    void emergencyStop(int elevatorId);

    /**
     * Get system health metrics
     * @return Performance metrics (avg wait time, etc.)
     */
    SystemMetrics getMetrics();
}

### 5.4.2 Internal API (Scheduler Interface)

/**
 * Scheduler interface for request assignment
 */
public interface IScheduler {

    /**
     * Assign request to best elevator
     * @param request External or internal request
     * @return ID of assigned elevator
     */
    int assignElevator(Request request);

    /**
     * Get optimal route for an elevator
     * @param elevatorId Which elevator
     * @return Ordered list of floors to visit
     */
    List<Integer> getOptimalRoute(int elevatorId);

    /**
     * Update elevator position (called by elevator controller)
     * @param elevatorId Which elevator
     * @param currentFloor Current position
     * @param state Current state
     */
    void updateElevatorState(int elevatorId, int currentFloor, State state);

    /**
     * Add request to queue
     * @param request Request to add
     */
    void enqueueRequest(Request request);

    /**
     * Remove fulfilled request
     * @param requestId Request to remove
     */
    void dequeueRequest(String requestId);
}

## 6. Low-Level Object-Oriented Design

### 6.1 Core Classes

### 6.1.1 Enumerations

/**
 * Direction of elevator movement or passenger intent
 */
public enum Direction {
    UP,
    DOWN,
    IDLE
}

/**
 * Elevator operational state
 */
public enum State {
    STATIONARY,  // Not moving, doors may be open or closed
    MOVING,      // In transit between floors
    OUTOFORDER   // Maintenance mode, unavailable
}

/**
 * Type of request
 */
public enum RequestType {
    EXTERNAL,  // From floor button (has direction)
    INTERNAL   // From cabin button (no direction, only destination)
}

### 6.1.2 Request Class

/**
 * Represents a request for elevator service
 */
public class Request {
    private String id;
    private int sourceFloor;
    private int destinationFloor;  // -1 if external request
    private Direction direction;
    private RequestType type;
    private long timestamp;

    // External request (from floor)
    public Request(int sourceFloor, Direction direction) {
        this.id = UUID.randomUUID().toString();
        this.sourceFloor = sourceFloor;
        this.destinationFloor = -1;
        this.direction = direction;
        this.type = RequestType.EXTERNAL;
        this.timestamp = System.currentTimeMillis();
    }

    // Internal request (from cabin)
    public Request(int sourceFloor, int destinationFloor) {
        this.id = UUID.randomUUID().toString();
        this.sourceFloor = sourceFloor;
        this.destinationFloor = destinationFloor;
        this.direction = (destinationFloor > sourceFloor) ? Direction.UP : Direction.DOWN;
        this.type = RequestType.INTERNAL;
        this.timestamp = System.currentTimeMillis();
    }

    // Getters
    public String getId() { return id; }
    public int getSourceFloor() { return sourceFloor; }
    public int getDestinationFloor() { return destinationFloor; }
    public Direction getDirection() { return direction; }
    public RequestType getType() { return type; }
    public long getTimestamp() { return timestamp; }
}

### 6.1.3 Elevator Class

/**
 * Represents a single elevator with its state and behavior
 */
public class Elevator {
    private int id;
    private int currentFloor;
    private State state;
    private Direction direction;
    private int capacity;  // Max persons
    private int currentLoad;

    // Separate queues for up and down requests
    private PriorityQueue<Integer> upQueue;    // Min heap (ascending)
    private PriorityQueue<Integer> downQueue;  // Max heap (descending)

    private Display display;

    public Elevator(int id, int capacity) {
        this.id = id;
        this.currentFloor = 0;  // Start at ground floor
        this.state = State.STATIONARY;
        this.direction = Direction.IDLE;
        this.capacity = capacity;
        this.currentLoad = 0;

        this.upQueue = new PriorityQueue<>();  // Natural ordering (ascending)
        this.downQueue = new PriorityQueue<>(Collections.reverseOrder());  // Descending
        this.display = new Display();
    }

    /**
     * Add a floor to this elevator's route
     */
    public void addStop(int floor) {
        if (floor > currentFloor) {
            upQueue.add(floor);
        } else if (floor < currentFloor) {
            downQueue.add(floor);
        }
        // If floor == currentFloor, ignore (already here)

        // If idle, start moving
        if (direction == Direction.IDLE) {
            if (!upQueue.isEmpty()) {
                direction = Direction.UP;
                state = State.MOVING;
            } else if (!downQueue.isEmpty()) {
                direction = Direction.DOWN;
                state = State.MOVING;
            }
        }
    }

    /**
     * Move elevator one floor in current direction
     */
    public void move() {
        if (state != State.MOVING) return;

        if (direction == Direction.UP) {
            currentFloor++;
            checkArrival();
        } else if (direction == Direction.DOWN) {
            currentFloor--;
            checkArrival();
        }

        display.update(currentFloor, direction);
    }

    /**
     * Check if we've arrived at a scheduled stop
     */
    private void checkArrival() {
        boolean shouldStop = false;

        if (direction == Direction.UP && !upQueue.isEmpty()) {
            if (upQueue.peek() == currentFloor) {
                shouldStop = true;
                upQueue.poll();
            }
        } else if (direction == Direction.DOWN && !downQueue.isEmpty()) {
            if (downQueue.peek() == currentFloor) {
                shouldStop = true;
                downQueue.poll();
            }
        }

        if (shouldStop) {
            stop();
        }
    }

    /**
     * Stop at current floor
     */
    public void stop() {
        state = State.STATIONARY;
        openDoors();

        // After passengers exit/enter, determine next direction
        scheduleNextMove();
    }

    /**
     * Determine next direction based on pending requests
     * Implements LOOK algorithm logic
     */
    private void scheduleNextMove() {
        // Continue in same direction if more stops ahead
        if (direction == Direction.UP && !upQueue.isEmpty()) {
            state = State.MOVING;
            // Keep direction UP
        } else if (direction == Direction.DOWN && !downQueue.isEmpty()) {
            state = State.MOVING;
            // Keep direction DOWN
        }
        // Reverse direction if requests in opposite direction
        else if (direction == Direction.UP && !downQueue.isEmpty()) {
            direction = Direction.DOWN;
            state = State.MOVING;
        } else if (direction == Direction.DOWN && !upQueue.isEmpty()) {
            direction = Direction.UP;
            state = State.MOVING;
        }
        // No more requests - become idle
        else {
            direction = Direction.IDLE;
            state = State.STATIONARY;
        }
    }

    /**
     * Check if this elevator can efficiently serve a request
     * Used by scheduler for elevator assignment
     */
    public boolean canTakeRequest(Request request) {
        if (state == State.OUTOFORDER) return false;
        if (currentLoad >= capacity) return false;

        int requestFloor = request.getSourceFloor();
        Direction requestDir = request.getDirection();

        // Case 1: Elevator is idle - always can take
        if (direction == Direction.IDLE) return true;

        // Case 2: Moving toward request in same direction - ideal
        if (direction == Direction.UP && requestDir == Direction.UP && requestFloor > currentFloor) {
            return true;
        }
        if (direction == Direction.DOWN && requestDir == Direction.DOWN && requestFloor < currentFloor) {
            return true;
        }

        // Case 3: Will eventually serve (after reversing) - acceptable
        // This is handled by scheduler's cost calculation

        return false;
    }

    /**
     * Calculate cost/score for serving a request
     * Lower score = better candidate
     */
    public int calculateCost(Request request) {
        int requestFloor = request.getSourceFloor();
        Direction requestDir = request.getDirection();

        // Out of order - infinite cost
        if (state == State.OUTOFORDER) return Integer.MAX_VALUE;

        // Idle - cost is just distance
        if (direction == Direction.IDLE) {
            return Math.abs(currentFloor - requestFloor);
        }

        // Moving towards in same direction - low cost
        if (direction == requestDir) {
            if (direction == Direction.UP && requestFloor > currentFloor) {
                return requestFloor - currentFloor;  // Direct path
            }
            if (direction == Direction.DOWN && requestFloor < currentFloor) {
                return currentFloor - requestFloor;  // Direct path
            }
        }

        // Moving towards but opposite direction - medium cost
        // Need to drop current passengers first, then serve request
        if (direction == Direction.UP && requestFloor < currentFloor) {
            int toTop = getHighestStop() - currentFloor;
            int toRequest = getHighestStop() - requestFloor;
            return toTop + toRequest + 50;  // Penalty for direction change
        }
        if (direction == Direction.DOWN && requestFloor > currentFloor) {
            int toBottom = currentFloor - getLowestStop();
            int toRequest = requestFloor - getLowestStop();
            return toBottom + toRequest + 50;  // Penalty
        }

        // Moving away - high cost
        return 1000;
    }

    private int getHighestStop() {
        return upQueue.isEmpty() ? currentFloor : Collections.max(upQueue);
    }

    private int getLowestStop() {
        return downQueue.isEmpty() ? currentFloor : Collections.min(downQueue);
    }

    private void openDoors() {
        // Door opening logic
        System.out.println("Elevator " + id + ": Doors opening at floor " + currentFloor);
    }

    // Getters
    public int getId() { return id; }
    public int getCurrentFloor() { return currentFloor; }
    public State getState() { return state; }
    public Direction getDirection() { return direction; }
    public int getCurrentLoad() { return currentLoad; }
    public boolean isEmpty() { return upQueue.isEmpty() && downQueue.isEmpty(); }
}

### 6.1.4 Display Class

/**
 * Manages floor and direction displays
 */
public class Display {
    private int currentFloor;
    private Direction direction;

    public void update(int floor, Direction dir) {
        this.currentFloor = floor;
        this.direction = dir;
        render();
    }

    private void render() {
        String arrow = (direction == Direction.UP) ? "↑" :
                      (direction == Direction.DOWN) ? "↓" : "•";
        System.out.println("Display: Floor " + currentFloor + " " + arrow);
    }
}

### 6.1.5 Floor Class

/**
 * Represents a single floor with its controls
 */
public class Floor {
    private int floorNumber;
    private Display display;

    public Floor(int number) {
        this.floorNumber = number;
        this.display = new Display();
    }

    /**
     * Called when passenger presses UP or DOWN button
     */
    public void callElevator(Direction direction) {
        Request request = new Request(floorNumber, direction);
        ElevatorSystem.getInstance().requestElevator(floorNumber, direction);
    }

    public int getFloorNumber() {
        return floorNumber;
    }
}

### 6.1.6 Building Class

/**
 * Represents the building with all floors and elevators
 */
public class Building {
    private Floor[] floors;
    private Elevator[] elevators;
    private int totalFloors;
    private int totalElevators;

    public Building(int numFloors, int numElevators, int elevatorCapacity) {
        this.totalFloors = numFloors;
        this.totalElevators = numElevators;

        // Initialize floors (including ground floor at index 0)
        floors = new Floor[numFloors + 1];
        for (int i = 0; i <= numFloors; i++) {
            floors[i] = new Floor(i);
        }

        // Initialize elevators
        elevators = new Elevator[numElevators];
        for (int i = 0; i < numElevators; i++) {
            elevators[i] = new Elevator(i, elevatorCapacity);
        }
    }

    public Elevator getElevator(int id) {
        return elevators[id];
    }

    public Elevator[] getElevators() {
        return elevators;
    }

    public Floor getFloor(int number) {
        return floors[number];
    }

    public int getTotalFloors() {
        return totalFloors;
    }

    public int getTotalElevators() {
        return totalElevators;
    }
}

### 6.1.7 Scheduler Class

/**
 * Core scheduling logic - implements LOOK algorithm
 */
public class Scheduler {
    private Building building;
    private Queue<Request> requestQueue;

    public Scheduler(Building building) {
        this.building = building;
        this.requestQueue = new LinkedList<>();
    }

    /**
     * Main scheduling method - assigns request to best elevator
     */
    public int assignElevator(Request request) {
        Elevator[] elevators = building.getElevators();
        int bestElevator = -1;
        int lowestCost = Integer.MAX_VALUE;

        // Evaluate each elevator
        for (int i = 0; i < elevators.length; i++) {
            Elevator elevator = elevators[i];

            int cost = elevator.calculateCost(request);

            if (cost < lowestCost) {
                lowestCost = cost;
                bestElevator = i;
            }
        }

        // Assign request to best elevator
        if (bestElevator != -1) {
            elevators[bestElevator].addStop(request.getSourceFloor());
        }

        return bestElevator;
    }

    public void enqueueRequest(Request request) {
        requestQueue.add(request);
        assignElevator(request);
    }
}

### 6.1.8 ElevatorSystem Class (Singleton)

/**
 * Main system controller - Singleton pattern
 */
public class ElevatorSystem {
    private static ElevatorSystem instance;
    private Building building;
    private Scheduler scheduler;

    private ElevatorSystem(int floors, int elevators, int capacity) {
        this.building = new Building(floors, elevators, capacity);
        this.scheduler = new Scheduler(building);
    }

    public static ElevatorSystem getInstance() {
        if (instance == null) {
            // Default configuration for our building
            instance = new ElevatorSystem(20, 5, 15);
        }
        return instance;
    }

    /**
     * Public API: Request elevator from floor
     */
    public String requestElevator(int floor, Direction direction) {
        Request request = new Request(floor, direction);
        scheduler.enqueueRequest(request);
        return request.getId();
    }

    /**
     * Public API: Select destination from inside elevator
     */
    public void selectDestination(int elevatorId, int destinationFloor) {
        Elevator elevator = building.getElevator(elevatorId);
        elevator.addStop(destinationFloor);
    }

    /**
     * Public API: Get elevator status
     */
    public ElevatorStatus getStatus(int elevatorId) {
        Elevator elevator = building.getElevator(elevatorId);
        return new ElevatorStatus(
            elevator.getId(),
            elevator.getCurrentFloor(),
            elevator.getState(),
            elevator.getDirection(),
            elevator.getCurrentLoad()
        );
    }

    /**
     * Run elevator simulation
     */
    public void run() {
        // Main event loop - moves all elevators
        while (true) {
            for (Elevator elevator : building.getElevators()) {
                if (elevator.getState() == State.MOVING) {
                    elevator.move();
                }
            }

            try {
                Thread.sleep(1000);  // 1 second per floor (simplified)
            } catch (InterruptedException e) {
                break;
            }
        }
    }
}

### 6.2 Class Diagram

┌─────────────────────────────────────────────────────────┐
│              ElevatorSystem (Singleton)                 │
├─────────────────────────────────────────────────────────┤
│ - instance: ElevatorSystem (static)                     │
│ - building: Building                                    │
│ - scheduler: Scheduler                                  │
├─────────────────────────────────────────────────────────┤
│ + getInstance(): ElevatorSystem                         │
│ + requestElevator(floor, dir): String                   │
│ + selectDestination(elevId, floor): void                │
│ + getStatus(elevId): ElevatorStatus                     │
└────────────────┬────────────────────────────────────────┘
                 │
                 │ manages
                 ▼
┌────────────────────────────┐         ┌──────────────────┐
│        Building            │         │    Scheduler     │
├────────────────────────────┤         ├──────────────────┤
│ - floors: Floor[]          │◄────────│ - building       │
│ - elevators: Elevator[]    │         │ - requestQueue   │
│ - totalFloors: int         │         ├──────────────────┤
│ - totalElevators: int      │         │ + assignElevator │
├────────────────────────────┤         │ + enqueueRequest │
│ + getElevator(id)          │         └──────────────────┘
│ + getFloor(num)            │
└──────┬─────────────────────┘
       │
       │ contains
       ▼
┌────────────────────────────────────────────────────┐
│                  Elevator                          │
├────────────────────────────────────────────────────┤
│ - id: int                                          │
│ - currentFloor: int                                │
│ - state: State                                     │
│ - direction: Direction                             │
│ - capacity: int                                    │
│ - currentLoad: int                                 │
│ - upQueue: PriorityQueue<Integer>                  │
│ - downQueue: PriorityQueue<Integer>                │
│ - display: Display                                 │
├────────────────────────────────────────────────────┤
│ + addStop(floor): void                             │
│ + move(): void                                     │
│ + stop(): void                                     │
│ + canTakeRequest(request): boolean                 │
│ + calculateCost(request): int                      │
└────────────────────────────────────────────────────┘
       │
       │ has-a
       ▼
┌────────────────────┐       ┌────────────────────────┐
│     Display        │       │       Floor            │
├────────────────────┤       ├────────────────────────┤
│ - currentFloor     │       │ - floorNumber: int     │
│ - direction        │       │ - display: Display     │
├────────────────────┤       ├────────────────────────┤
│ + update()         │       │ + callElevator(dir)    │
│ + render()         │       └────────────────────────┘
└────────────────────┘

┌─────────────┐    ┌──────────┐    ┌──────────────────┐
│  Direction  │    │  State   │    │    Request       │
├─────────────┤    ├──────────┤    ├──────────────────┤
│ UP          │    │STATIONARY│    │ - id: String     │
│ DOWN        │    │ MOVING   │    │ - sourceFloor    │
│ IDLE        │    │OUTOFORDER│    │ - destFloor      │
└─────────────┘    └──────────┘    │ - direction      │
                                   │ - type           │
                                   │ - timestamp      │
                                   └──────────────────┘

### 6.3 Design Patterns Used

Singleton Pattern: ElevatorSystem - Only one instance of the system

Strategy Pattern: Scheduler can use different algorithms (FCFS, SSTF, SCAN, LOOK)

State Pattern: Elevator has clear states (STATIONARY, MOVING, OUT_OF_ORDER)

Observer Pattern: Display observes elevator position changes

Queue Pattern: Priority queues for efficient floor management

## 7. Scheduling Algorithms

Elevator scheduling is analogous to disk scheduling in operating systems. We analyze four algorithms:

### 7.1 First Come First Serve (FCFS)

### Algorithm:

Serve requests in the exact order they arrive (FIFO queue).

### Example:

Building: 11 floors (0-10)
Elevator at floor 5

Request Queue (arrival order):
1. Floor 2, DOWN
2. Floor 4, UP
3. Floor 3, DOWN
4. Floor 6, UP
5. Floor 8, DOWN

Elevator path (FCFS):
5 → 2 → 4 → 3 → 6 → 8
Total distance: 3+2+1+3+2 = 11 floors

### Implementation:

public class FCFSScheduler implements IScheduler {
    private Queue<Request> queue = new LinkedList<>();

    @Override
    public int assignElevator(Request request) {
        queue.add(request);
        // Always assign to first available elevator
        for (Elevator e : elevators) {
            if (e.isEmpty()) return e.getId();
        }
        return 0;  // Default to elevator 0
    }
}

### Analysis:

Advantages:

✅ Simple to implement

✅ Fair - no starvation (all requests eventually served)

✅ Predictable behavior

Disadvantages:

❌ High average wait time

❌ Inefficient movement (excessive back-and-forth)

❌ Cannot serve multiple requests in parallel

❌ Poor performance during high load

Time Complexity: O(1) for insertion, O(1) for retrieval

Use Case: Only suitable for very low-traffic buildings

### 7.2 Shortest Seek Time First (SSTF)

### Algorithm:

Always serve the request closest to elevator's current position.

### Example:

Elevator at floor 5

Requests with distances:
- Floor 2, DOWN  (distance: 3)
- Floor 4, UP    (distance: 1) ← SERVED FIRST
- Floor 3, DOWN  (distance: 2)
- Floor 6, UP    (distance: 1)
- Floor 8, DOWN  (distance: 3)

Elevator path (SSTF):
5 → 4 → 6 → 3 → 2 → 8
(Serves by proximity, not arrival order)
Total distance: 1+2+3+1+6 = 13 floors
Wait times vary significantly

### Implementation:

public class SSTFScheduler implements IScheduler {
    @Override
    public int assignElevator(Request request) {
        int minDistance = Integer.MAX_VALUE;
        int bestElevator = 0;

        for (Elevator e : elevators) {
            int distance = Math.abs(e.getCurrentFloor() - request.getSourceFloor());
            if (distance < minDistance) {
                minDistance = distance;
                bestElevator = e.getId();
            }
        }

        return bestElevator;
    }
}

### Analysis:

Advantages:

✅ Lower average response time than FCFS

✅ Reduced elevator movement

✅ Better throughput

Disadvantages:

❌ Starvation possible - requests far from elevator may wait indefinitely

❌ Unfair to distant floors

❌ Still cannot serve multiple requests efficiently in one direction

Time Complexity: O(n) to find minimum distance

Use Case: Medium-traffic buildings with uniform request distribution

### 7.3 SCAN (Elevator Algorithm)

### Algorithm:

Elevator moves in one direction, serving all requests in its path until reaching the end, then reverses direction.

### Example:

Elevator at floor 3, moving UP

Requests:
- Floor 5, UP
- Floor 9, UP
- Floor 4, DOWN
- Floor 2, DOWN

Elevator path (SCAN):
3 → 4 → 5 (stop) → 6 → 7 → 8 → 9 (stop) → 10 (top, forced)
    → 9 (drop) → 8 → 7 → 6 → 5 (drop) → 4 (stop) → 3 → 2 (stop)

Visualization:
Floor 10 │         ↑ (forced to top)
Floor 9  │     ↑  ↓ (pick up, drop down)
Floor 5  │   ↑   ↓ (pick up, drop down)
Floor 4  │ ↑     ↓ (pick down)
Floor 3  │ ↑       (start)
Floor 2  │       ↓ (drop)

Total distance: 7 up + 8 down = 15 floors
Multiple requests served in single pass

### Implementation:

public class SCANScheduler implements IScheduler {
    @Override
    public int assignElevator(Request request) {
        // Find elevator moving in request direction
        for (Elevator e : elevators) {
            if (e.getDirection() == request.getDirection() &&
                isMovingTowards(e, request)) {
                return e.getId();
            }
        }
        // Otherwise, find closest idle elevator
        return findClosestIdleElevator(request);
    }

    private boolean isMovingTowards(Elevator e, Request req) {
        if (e.getDirection() == Direction.UP) {
            return req.getSourceFloor() > e.getCurrentFloor();
        } else {
            return req.getSourceFloor() < e.getCurrentFloor();
        }
    }
}

### Analysis:

Advantages:

✅ Serves multiple requests in parallel (key advantage)

✅ No starvation - all floors visited in each cycle

✅ Predictable pattern

✅ Good for high-traffic scenarios

Disadvantages:

❌ Unnecessary travel to end floor even without requests

❌ Higher wait time for floors just passed (must wait full cycle)

❌ Continuous movement wastes energy

Time Complexity: O(n) where n = number of floors

Use Case: High-traffic buildings, peak hours

### 7.4 LOOK (Optimized SCAN) ⭐ RECOMMENDED

### Algorithm:

Similar to SCAN, but reverses at the last request in current direction, not at the last floor.

### Example:

Same scenario as SCAN:
Elevator at floor 3, moving UP

Requests:
- Floor 5, UP
- Floor 9, UP
- Floor 4, DOWN
- Floor 2, DOWN

Elevator path (LOOK):
3 → 4 → 5 (stop) → 6 → 7 → 8 → 9 (stop, REVERSE - last request)
    → 8 → 7 → 6 → 5 (drop) → 4 (stop) → 3 → 2 (stop, REVERSE - last request)

Comparison with SCAN:
SCAN:  3 → 5 → 9 → 10 (forced) → 9 → 5 → 4 → 2
LOOK:  3 → 5 → 9 (reverse) → 5 → 4 → 2

Savings: Eliminated 9 → 10 → 9 (2 floors saved per direction)

### Implementation (Full):

public class LOOKScheduler implements IScheduler {

    @Override
    public int assignElevator(Request request) {
        int bestElevator = -1;
        int bestScore = Integer.MAX_VALUE;

        for (Elevator e : elevators) {
            if (e.getState() == State.OUTOFORDER) continue;

            int score = calculateScore(e, request);
            if (score < bestScore) {
                bestScore = score;
                bestElevator = e.getId();
            }
        }

        return bestElevator;
    }

    /**
     * Calculate assignment score based on elevator state
     * Lower score = better candidate
     */
    private int calculateScore(Elevator elevator, Request request) {
        Direction elevDir = elevator.getDirection();
        Direction reqDir = request.getDirection();
        int elevFloor = elevator.getCurrentFloor();
        int reqFloor = request.getSourceFloor();

        // CASE 1: Elevator is IDLE (BEST)
        if (elevDir == Direction.IDLE) {
            return Math.abs(elevFloor - reqFloor);
        }

        // CASE 2: Moving TOWARDS request, SAME direction (EXCELLENT)
        if (elevDir == reqDir) {
            if (elevDir == Direction.UP && reqFloor > elevFloor) {
                return reqFloor - elevFloor;  // Low score
            }
            if (elevDir == Direction.DOWN && reqFloor < elevFloor) {
                return elevFloor - reqFloor;  // Low score
            }
        }

        // CASE 3: Moving TOWARDS request, OPPOSITE direction (ACCEPTABLE)
        if (elevDir == Direction.UP && reqDir == Direction.DOWN && reqFloor < elevFloor) {
            // Will serve after reaching highest request and reversing
            int toTop = elevator.getHighestStop() - elevFloor;
            int toReq = elevator.getHighestStop() - reqFloor;
            return toTop + toReq + 100;  // Moderate penalty
        }
        if (elevDir == Direction.DOWN && reqDir == Direction.UP && reqFloor > elevFloor) {
            int toBottom = elevFloor - elevator.getLowestStop();
            int toReq = reqFloor - elevator.getLowestStop();
            return toBottom + toReq + 100;  // Moderate penalty
        }

        // CASE 4: Moving AWAY (WORST)
        return 10000;  // High penalty
    }
}

### Analysis:

Advantages:

✅ Eliminates unnecessary travel (vs SCAN)

✅ Serves multiple requests in parallel

✅ No starvation

✅ Energy efficient (stops when no requests)

✅ Best all-around algorithm for most scenarios

Disadvantages:

⚠️ Slightly more complex than SCAN (requires tracking last request)

Time Complexity: O(n) where n = number of pending requests

Use Case: ⭐ RECOMMENDED for all buildings - Best balance of efficiency, fairness, and energy savings

### 7.5 Algorithm Comparison

| Algorithm | Avg Wait Time | Movement Efficiency | Starvation Risk | Parallel Serving | Best For |
| --- | --- | --- | --- | --- | --- |
| **FCFS** | High | Poor | None | No | Low traffic only |
| **SSTF** | Medium | Good | **High** | No | Uniform traffic |
| **SCAN** | Medium | Good | None | **Yes** | High traffic peaks |
| **LOOK** | **Low** | **Excellent** | None | **Yes** | **All scenarios** ⭐ |

### 7.6 Real-World Considerations

For our 20-floor building:

Primary choice: LOOK algorithm

Reason: Handles both office (concentrated) and residential (distributed) traffic patterns efficiently

Optimization: Dynamic elevator grouping during peak hours

## 8. Elevator Count Determination

### 8.1 Industry Performance Metrics

### Key Metrics:

1. Round Trip Time (RTT)

Time for elevator to complete full cycle: lobby → serve requests → return

Formula:

`

RTT = (2 × H / S) + (N_stops + 1) × T_door + (2 × N_stops × P × T_transfer)

Where:

H = Height to serve (meters)

S = Elevator speed (m/s)

N_stops = Average number of stops

T_door = Door operation time (seconds)

P = Average passengers per trip

T_transfer = Passenger transfer time (sec/person)

`

2. Interval (INT)

Time between successive elevator arrivals at lobby

Formula: Interval = RTT / N_elevators

Target: 25-40 seconds (good to excellent service)

3. Handling Capacity (HC)

Percentage of population transported in 5 minutes

Formula: HC = (300 / RTT) × Capacity × N_elevators / Population × 100%

Target: 15-20% for office buildings

4. Average Wait Time (AWT)

Theoretical minimum: AWT ≈ Interval / 2

Target: < 30 seconds

### 8.2 Calculations for Our Building

### Given Parameters:

Building height: 70 meters (20 floors × 3.5m)
Elevator speed: 2.5 m/s
Floor height: 3.5 meters
Elevator capacity: 15 passengers (practical)
Door operation: 6 seconds (3s open + 3s close)
Passenger transfer: 0.8 sec/person

### Step 1: Calculate Expected Number of Stops

For office up-peak traffic:

Probability of NO stop at a floor: (1 - 1/N_floors)^P
Expected stops: N_floors × [1 - (1 - 1/N_floors)^P]

For our office zone (10 floors, 14 passengers):
N_stops = 10 × [1 - (1 - 1/10)^14]
        = 10 × [1 - 0.229]
        = 7.71 ≈ 7-8 stops

Use 7 stops for calculation.

### Step 2: Calculate RTT (Office Zone)

Office zone: Floors 1-10 (35 meters)

RTT = (2 × 35 / 2.5) + (7 + 1) × 6 + (2 × 7 × 12 × 0.8)
    = 28 + 48 + 134.4
    = 210.4 seconds

With LOOK algorithm optimization (~15% improvement):

RTT_effective = 210 × 0.85 = 178.5 seconds ≈ 180 seconds

### Step 3: Calculate Required Elevators

Method 1: Interval Target

Target: 35 seconds (good service)

N_elevators = RTT / Interval
            = 180 / 35
            = 5.14 ≈ 5 elevators

Method 2: Handling Capacity

Peak demand: 112 passengers in 5 minutes (morning office peak)

HC = (300 / RTT) × Capacity × N_elevators / Peak_Population

For 5 elevators:
HC = (300 / 180) × 15 × 5 / 112
   = 125 / 112
   = 111.6% ✓ (exceeds 100%, excellent)

For 4 elevators:
HC = (300 / 180) × 15 × 4 / 112
   = 100 / 112
   = 89.3% ⚠️ (below 100%, marginal)

### 8.3 Residential Peak Analysis

Residential peak: 60 passengers in 5 minutes (evening)

Residential zone RTT: Floors 11-20 (high-rise)
Travel distance: 35 meters (from ground to floor 20)

RTT_residential = (2 × 35 / 2.5) + (6 × 6) + (2 × 6 × 12 × 0.8)
                = 28 + 36 + 115.2
                = 179.2 ≈ 180 seconds

With LOOK: RTT_eff = 180 × 0.85 = 153 seconds

For 5 elevators (3 allocated to residential during evening):

Interval = 153 / 3 = 51 seconds (acceptable for residential)
HC = (300 / 153) × 15 × 3 / 60 = 147% ✓ (excellent)

For 6 elevators (4 allocated to residential):

Interval = 153 / 4 = 38 seconds (good service)
HC = (300 / 153) × 15 × 4 / 60 = 196% ✓ (excellent)

### 8.4 Summary Table

| Configuration | Office Interval | Office HC | Residential Interval | Residential HC | Recommendation |
| --- | --- | --- | --- | --- | --- |
| **3 Elevators** | 60s | 67% | 76s | 98% | ❌ Insufficient |
| **4 Elevators** | 45s | 89% | 57s | 131% | ⚠️ Marginal for office |
| **5 Elevators** | **36s** | **111%** | **51s** | **147%** | ✅ **Recommended Minimum** |
| **6 Elevators** | 30s | 134% | 38s | 196% | ✅ **Premium Service** |
| **7 Elevators** | 26s | 156% | 33s | 229% | ❌ Over-provisioned |

## 9. Technical Recommendations

### 9.1 Primary Recommendation: 5 Elevators

Configuration:

Quantity: 5 elevators

Algorithm: LOOK with dynamic zoning

Capacity: 1,600 kg (21 persons rated, 15 practical)

Speed: 2.5 m/s

Control: Smart group control system

Performance:

Office peak interval: 36 seconds (good)

Residential peak interval: 51 seconds (acceptable)

Handling capacity: 111% office / 147% residential

System redundancy: 20% (4 elevators still functional if one fails)

Zoning Strategy:

MORNING PEAK (8:00-10:00 AM) - Office Arrivals
┌────────────────────────────────────┐
│ Elevators 1-3: Office Zone (1-10)  │
│ Elevators 4-5: Residential + Backup│
└────────────────────────────────────┘

MIDDAY (10:00 AM - 5:00 PM) - Distributed Traffic
┌────────────────────────────────────┐
│ All 5 Elevators: Dynamic LOOK      │
│ Serve all floors based on demand   │
└────────────────────────────────────┘

EVENING PEAK (5:00-8:00 PM) - Dual Peak
┌────────────────────────────────────┐
│ Elevators 1-2: Office Departure     │
│ Elevators 3-5: Residential Arrival  │
└────────────────────────────────────┘

Rationale:

✅ Meets performance targets for both office and residential

✅ 20% redundancy ensures reliability

✅ Handles 500 residential occupants adequately

✅ Good balance for standard mid-rise buildings

Limitations:

Residential interval (51s) is acceptable but not premium

During simultaneous peaks, service may be stretched

### 9.2 Premium Recommendation: 6 Elevators

Configuration:

Quantity: 6 elevators

Algorithm: LOOK with enhanced zoning

Specifications: Same as 5-elevator option

Performance:

Office peak interval: 30 seconds (excellent)

Residential peak interval: 38 seconds (good)

Handling capacity: 134% office / 196% residential

System redundancy: 16%

Enhanced Zoning:

MORNING PEAK (8:00-10:00 AM)
┌────────────────────────────────────┐
│ Elevators 1-4: Office Zone          │
│ Elevators 5-6: Residential + Backup │
└────────────────────────────────────┘

EVENING PEAK (5:00-8:00 PM)
┌────────────────────────────────────┐
│ Elevators 1-2: Office Departure     │
│ Elevators 3-6: Residential Arrival  │
│ (4 elevators for 500 residents)     │
└────────────────────────────────────┘

Rationale:

✅ Excellent service quality for both user groups

✅ Comfortably handles 500 residential occupants

✅ Superior user experience for premium buildings

✅ Better handles concurrent peaks

Best For:

Premium/luxury residential buildings

High tenant expectations

Buildings where concurrent office/residential peaks occur

### 9.3 Budget Alternative: 4 Elevators with Destination Dispatch

Configuration:

Quantity: 4 elevators

Algorithm: LOOK + Destination Dispatch System

Speed: 2.5-3.0 m/s (higher speed to compensate)

Control: AI-powered destination dispatch

Destination Dispatch Technology:

Lobby kiosks for destination entry

Passengers grouped by destination before boarding

Reduces stops per trip through intelligent batching

25-30% efficiency improvement over traditional systems

Mobile app integration possible

Performance (with 25% dispatch efficiency gain):

Effective RTT: 180 × 0.75 = 135 seconds

Office interval: 135 / 4 = 34 seconds (good)

Residential interval: 45 seconds (good)

HC: 123% office / 148% residential

Rationale:

✅ Technology compensates for fewer elevators

✅ Modern user experience

✅ Energy efficient

Risks:

⚠️ Higher technology dependency

⚠️ Requires user education

⚠️ 25% capacity loss if one elevator fails (tight)

⚠️ More complex maintenance

Best For:

Budget-constrained projects

Tech-savvy tenant base

Modern commercial buildings

### 9.4 Decision Framework

Choose 5 Elevators (Standard) if:

✅ Standard/mid-tier building class

✅ Balance of reliability and performance required

✅ Lower technology risk preferred

✅ Most versatile option

Choose 6 Elevators (Premium) if:

✅ Premium/luxury building

✅ 500 residential occupants need excellent service

✅ High tenant expectations

✅ Budget allows enhanced service

Choose 4 Elevators (Budget + Tech) if:

✅ Budget is primary constraint

✅ Willing to adopt destination dispatch technology

✅ Modern building with tech-savvy users

✅ Can accept higher technology risk

## 10. Scalability and Optimizations

### 10.1 Scalability Strategies

### Horizontal Scaling (Add More Elevators)

Design Considerations:

System architecture supports dynamic elevator addition

Scheduler automatically includes new elevators in assignment logic

No code changes required for fleet expansion

Example: Adding 7th Elevator

// Current system
ElevatorSystem system = ElevatorSystem.getInstance();

// Add new elevator dynamically
Elevator newElevator = new Elevator(6, 15);  // ID: 6, Capacity: 15
building.addElevator(newElevator);

// Scheduler automatically uses it
// No other changes needed

### Vertical Scaling (Optimize Individual Elevators)

Performance Improvements:

Faster elevators: Upgrade from 2.5 m/s to 3.0 m/s

- Reduces RTT by ~15%

- Better for taller buildings

Larger capacity: 1,600 kg → 2,000 kg (21 → 26 persons)

- Increases HC by ~25%

- Requires larger shaft (physical constraint)

Faster doors: 6s → 4s operation time

- Reduces RTT by ~20 seconds

- Significant impact on performance

### Population Growth Handling

Current design handles:

10-15% population growth without elevator addition

15-25% growth may require 6th elevator

Beyond 25%: Consider express elevators or zoning

### 10.2 Advanced Optimizations

### 1. Predictive Scheduling

Concept: Use historical data to pre-position elevators

public class PredictiveScheduler extends LOOKScheduler {
    private TrafficPredictor predictor;

    @Override
    public void prePositionElevators() {
        LocalTime now = LocalTime.now();

        // Morning pre-positioning (7:45 AM)
        if (now.isAfter(LocalTime.of(7, 45)) &&
            now.isBefore(LocalTime.of(8, 0))) {
            // Send 3 elevators to ground floor
            for (int i = 0; i < 3; i++) {
                elevators[i].addStop(0);
            }
        }

        // Evening pre-positioning (5:45 PM)
        if (now.isAfter(LocalTime.of(17, 45)) &&
            now.isBefore(LocalTime.of(18, 0))) {
            // Distribute elevators to office floors
            elevators[0].addStop(5);
            elevators[1].addStop(8);
            elevators[2].addStop(10);
        }
    }
}

Benefits:

20-30% reduction in wait time during peaks

Proactive rather than reactive

Improves user experience

### 2. Dynamic Capacity Adjustment

Concept: Adjust elevator allocation based on real-time demand

public class DynamicZoning {
    private int officeElevators;
    private int residentialElevators;

    public void adjustAllocation() {
        int officeLoad = getOfficeLoadFactor();
        int residentialLoad = getResidentialLoadFactor();

        if (officeLoad > residentialLoad * 2) {
            // Heavy office traffic
            officeElevators = 4;
            residentialElevators = 1;
        } else if (residentialLoad > officeLoad * 2) {
            // Heavy residential traffic
            officeElevators = 2;
            residentialElevators = 3;
        } else {
            // Balanced
            officeElevators = 3;
            residentialElevators = 2;
        }
    }
}

### 3. Energy Optimization

Strategies:

Sleep mode: Idle elevators power down non-essential systems

Regenerative braking: Capture energy during descent

Load-based speed: Adjust speed based on passenger count

Off-peak reduction: Reduce active elevators during low traffic

Example:

public class EnergyOptimizer {
    public void optimizeFleet() {
        int activeRequests = requestQueue.size();

        if (activeRequests < 5 && isOffPeak()) {
            // Put 2 elevators to sleep
            elevators[3].setState(State.SLEEP);
            elevators[4].setState(State.SLEEP);
        }
    }

    private boolean isOffPeak() {
        LocalTime now = LocalTime.now();
        return now.isAfter(LocalTime.of(22, 0)) ||
               now.isBefore(LocalTime.of(6, 0));
    }
}

### 10.3 Fault Tolerance

### Graceful Degradation

Scenario: One elevator fails

public class FaultHandler {
    public void handleElevatorFailure(int elevatorId) {
        Elevator failed = elevators[elevatorId];
        failed.setState(State.OUTOFORDER);

        // Redistribute requests
        Queue<Integer> pendingStops = failed.getAllStops();
        for (int floor : pendingStops) {
            Request newReq = new Request(floor, Direction.UP);
            scheduler.assignElevator(newReq);
        }

        // Alert monitoring system
        monitoringSystem.alert("Elevator " + elevatorId + " failed");

        // System continues with N-1 elevators
        // Performance degrades gracefully:
        // 5 elevators → 4 elevators: Interval 36s → 45s (still acceptable)
    }
}

### Health Monitoring

public class HealthMonitor {
    private Map<Integer, ElevatorHealth> healthStatus;

    public void monitorElevators() {
        for (Elevator e : elevators) {
            ElevatorHealth health = new ElevatorHealth();
            health.doorCycles = e.getDoorCycleCount();
            health.tripCount = e.getTripCount();
            health.avgResponseTime = e.getAvgResponseTime();

            // Predictive maintenance
            if (health.doorCycles > 10000) {
                schedulePreventiveMaintenance(e.getId());
            }
        }
    }
}

### 10.4 Performance Monitoring

Key Metrics to Track:

public class PerformanceMetrics {
    // Real-time metrics
    private double avgWaitTime;
    private double avgTravelTime;
    private int totalTrips;
    private int peakLoad;

    // Per-elevator metrics
    private Map<Integer, Double> elevatorUtilization;

    // System health
    private double systemAvailability;  // Target: > 99.9%

    public void logMetrics() {
        System.out.println("=== Performance Report ===");
        System.out.println("Avg Wait Time: " + avgWaitTime + "s");
        System.out.println("Avg Travel Time: " + avgTravelTime + "s");
        System.out.println("Total Trips: " + totalTrips);
        System.out.println("System Uptime: " + systemAvailability + "%");

        // Alert if thresholds exceeded
        if (avgWaitTime > 30) {
            alert("Wait time exceeding target");
        }
    }
}

## Appendices

### Appendix A: Key Formulas

Round Trip Time (RTT):
RTT = (2H/S) + (N_stops + 1) × T_door + 2 × N_stops × P × T_transfer

Interval:
Interval = RTT / N_elevators

Handling Capacity (HC):
HC = (300 / RTT) × Capacity × N_elevators / Population × 100%

Expected Stops:
N_stops = N_floors × [1 - (1 - 1/N_floors)^P_passengers]

Average Wait Time:
AWT ≈ Interval / 2 (theoretical minimum)

### Appendix B: Technical Specifications

Building Parameters:

Total floors: 20

Floor height: 3.5 meters

Total height: 70 meters

Population: 1,620 persons (500 residential + 1,120 office)

Elevator Specifications:

Capacity: 1,600 kg (21 persons rated, 15 practical)

Speed: 2.5 m/s

Acceleration: 1.0 m/s²

Door width: 1,100 mm

Door operation: 6 seconds (3s open + 3s close)

Performance Targets:

Interval: 25-40 seconds

Handling Capacity: 15-20%

Average Wait Time: < 30 seconds

System Uptime: > 99.9%

### Appendix D: Glossary

RTT (Round Trip Time): Time for complete elevator cycle

Interval: Time between elevator arrivals at lobby

HC (Handling Capacity): % of population moved in 5 minutes

AWT (Average Wait Time): Average passenger wait time

LOOK Algorithm: Optimized SCAN that reverses at last request

Zoning: Dedicating elevators to specific floor ranges during peaks

Destination Dispatch: System where passengers pre-select floor before boarding